Software notes:

- run "egm.exe" - when application starts, automatically loads config file "config.egm",
to simplify constant operation (typing: service name, adres - they're the same, for the same place)

- now You can connenct EGM with PC on IrDA (or load test file "test.egm")

- please move EGM display - directly to PC IrDA port (small red box, under EGM display - it's IrDA interface)

- turn on EGM by pressing "POMIAR" key (after 1min stand by - EGM automatically turns off)

- PC will find new device - "EGM-2"
and Windows should automatically install standard driver: "Standard Modem over IR link"

- press "Connect" button in application window,
application automatically finds and lists, all serial ports (virtual) on PC

- select proper port (associated with PC IrDA port !!) and press "Test" button

- when communication will be established - button "OK" will change to enable (click it), if not - bad port was selected !! - please select different port, and try again (press "Test" again),
in case of problems: run "regedit.exe", and check port number manually:

- if successful - application automatically loads data from EGM, and fills the table

- now You can print results, or save them to a file

- Printing task adds result to "empty" bitmap file ("protokol.bmp") - exemplary report (obligatory in Poland),
of course - You can change this file, to version obligatory in Brasil